# Component Document Model v0.1 Amendment

## 1. Top-Level Object Syntax

Every textual document begins with:

component:<kind>

Version 0.1 defines four object kinds:

component:component
component:board
component:operation
component:schema

The identifier before : names the language family.

The identifier after : names the object kind.

## 2. Circuit Description

component:component CounterDemo {
    use standard.lib;

    device Clock is OSCILLATOR;
    device Counter is 74HC161;
    device R[4] is RESISTOR;
    device LED[4] is LED;

    Counter[Q0, Q1, Q2, Q3]
        as Counter.Q[0..3];

    connect Clock.OUT -> Counter.CLK;
    connect Counter.Q[0..3] -> R[0..3].1;
    connect R[0..3].2 -> LED[0..3].A;
    connect LED[0..3].K -> GND;

    probe Counter.Q[0..3];
}

component:component owns:

- Device instances

- logical and physical references

- collections

- connectivity

- probes associated with the machine

- machine-level virtual Devices

- topology

It does not own visual layout.

## 3. Board Description

component:board MainBoard {
    use CounterDemo;

    place Clock at 80, 180;
    place Counter at 320, 180;

    place R[0..3] at 560, 100
        direction vertical
        gap 24;

    place LED[0..3] at 760, 100
        direction vertical
        gap 24;

    route Clock.OUT -> Counter.CLK
        style orthogonal;

    route Counter.Q[0..3] -> R[0..3].1
        style bus;

    route R[0..3].2 -> LED[0..3].A
        style parallel;

    show port_name;
    show pin_number;
    show signal_value;

    hide power_wire;

    panel component at left;
    panel board at center;
    panel terminal at bottom;
}

component:board owns:

- Device placement

- size and orientation

- routes and wire presentation

- grouping for visual presentation

- labels

- visibility

- zoom and viewport defaults

- Board widgets

- panel arrangement

- display modes

- timeline and waveform placement

It must not redefine circuit topology.

## 4. Operation Description

component:operation NoiseTest {
    inject noise
        into Clock.OUT
        during tick 50..100;

    probe Clock.OUT;
    probe Counter.Q[0..3];

    run during tick 0..200;
}

The canonical machine form may still be JSON:

{
  "component:operation": {
    "name": "NoiseTest",
    "inject": {
      "noise": {
        "into": "Clock.OUT",
        "during": {
          "tick": "50..100"
        }
      }
    }
  }
}

component:operation owns:

- requests

- queries

- run control

- simulation steps

- injection

- inspection

- validation

- export

- probe requests

## 5. Schema Description

component:schema DeviceSchema {
    object device;

    require name;
    require type;
    require ports;

    optional package;
    optional timing;
    optional visual;
}

component:schema defines contracts for:

- Components

- Boards

- Operations

- Device Libraries

- Results

- external packages

The schema grammar remains provisional in v0.1.

## 6. Relationship Between Component and Board

component:component
        │
        │ describes the machine
        ▼
Components Platform
        │
        │ resolves and executes
        ▼
Resolved Component Model
        │
        │ is presented by
        ▼
component:board

The Board references the Component:

component:board MainBoard {
    use CounterDemo;
}

The Board must not duplicate:

- Device type

- port definition

- physical pin mapping

- connection semantics

- simulation behavior

The Board may describe how an existing connection is shown:

route Counter.Q[0..3] -> R[0..3].1
    style bus;

But it may not create a new circuit connection unless that interaction is translated into an Operation that modifies the Component.

## 7. File Organization

Recommended package:

CounterDemo/
├── main.component
├── main.board
├── operations/
│   └── noise_test.operation
└── component.json

main.component

component:component CounterDemo {
    ...
}

main.board

component:board MainBoard {
    use CounterDemo;
    ...
}

noise_test.operation

component:operation NoiseTest {
    ...
}

## 8. Multiple Objects in One File

Version 0.1 may permit one object per file for simplicity.

A future version may permit:

component:component CounterDemo {
    ...
}

component:board MainBoard {
    use CounterDemo;
    ...
}

component:operation DefaultRun {
    run during tick 0..200;
}

The parser should therefore treat each component:<kind> block as an independent top-level object.

## 9. Parser Rule

The first production becomes:

document
    := object*

object
    := "component" ":" object_kind identifier? block

object_kind
    := "component"
     | "board"
     | "operation"
     | "schema"

Examples:

component:component CounterDemo { }
component:board MainBoard { }
component:operation RunTest { }
component:schema DeviceSchema { }

## 10. JSON Envelope

Canonical JSON should mirror the textual notation:

{
  "component:component": {
    "name": "CounterDemo",
    "devices": [],
    "connections": []
  }
}

{
  "component:board": {
    "name": "MainBoard",
    "uses": "CounterDemo",
    "placements": [],
    "routes": []
  }
}

{
  "component:operation": {
    "name": "NoiseTest",
    "operations": []
  }
}

{
  "component:schema": {
    "name": "DeviceSchema",
    "object": "device"
  }
}

This keeps text and JSON structurally parallel.

## 11. Ownership Rule

component:component
    owns circuit meaning

component:board
    owns visual presentation

component:operation
    owns requests and experiments

component:schema
    owns contracts

No object kind may silently redefine another object’s responsibility.

## 12. Frozen v0.1 Decision

The following top-level syntax is frozen:

component:component { }
component:board { }
component:operation { }
component:schema { }

Named forms are also supported:

component:component CounterDemo { }
component:board MainBoard { }
component:operation NoiseTest { }
component:schema DeviceSchema { }

The colon : is reserved at the top level for identifying the Component object kind.