# Components Architecture Constitution

## 1. Vision

Components is a headless platform for electronic design, simulation, validation, learning, automation, and AI-assisted construction.

Components is not tied to any particular graphical interface, operating system, programming language, AI provider, or storage provider.

A user may interact with Components through:

- Command-line tools

- APIs

- AI agents

- Voice interfaces

- Python SDKs

- Web clients

- Stand-alone applications

- Tablet-first Boards

- IDE extensions

- Third-party clients

All clients operate on the same package model and receive the same validation and simulation results.

## 2. Core Principle

Components owns the truth. Boards own the experience.

Components Platform is authoritative for:

- Component definitions

- Circuit connectivity

- Package structure

- Pin resolution

- Dependency resolution

- Validation

- Simulation

- Export

- History

- Transactions

- Revisions

A Board is a replaceable client that presents and edits information through Components Platform.

No Board may define independent circuit semantics that differ from Components Platform.

## 3. User-Facing Model

A user works with three primary concepts:

### 3.1 Component

Component describes what the circuit is.

It defines:

- Inputs and outputs

- Chips and modules

- Connections

- Nets

- Buses

- Interfaces

- Hierarchy

- Functional relationships

Component does not define positions, routes, colors, zoom levels, or visual layout.

### 3.2 Board

Board describes how a Component is presented and interacted with.

It may define:

- Placement

- Grouping

- Alignment

- Routing

- Bus presentation

- Labels

- Portals

- Visibility

- Pin display

- Learning views

- Engineering views

- Breadboard views

- Timing views

- Debug views

A Board may not change electrical connectivity.

### 3.3 Resources

Resources describe what a package depends on and carries with it.

Resources may include:

- Component libraries

- Submodules

- Visual themes

- Custom symbols

- DIP package drawings

- Datasheets

- Lessons

- Images

- Audio

- Tests

- Examples

- Documentation

- Export profiles

Resources may affect availability and presentation, but visual resources may not redefine electrical truth.

## 4. Package

A Package is the portable document boundary of Components.

A Package contains or references:

- Component source

- One or more Boards

- Resources and dependencies

- Package metadata

- Tests

- Optional bundled modules

- Optional custom visuals

- Revision information

The user-facing package may appear as:

MyProject/
├── main.component
├── boards/
│   ├── learning.board
│   ├── engineering.board
│   └── breadboard.board
└── components.toml

The platform may internally maintain:

- Canonical IR

- Dependency lock data

- Simulation caches

- Validation indexes

- Generated Verilog

- Generated SystemVerilog

- Revision history

- Render models

These internal artifacts are platform-managed and are not primary user sources.

## 5. Component Language

Component is a concise, declarative, structural hardware language.

Its primary purpose is to express relationships rather than execution steps.

Example:

component CounterDemo {
    input Clock
    input Reset

    chip Counter : 74HC161
    chip LED[3:0] : LED

    connect Clock -> Counter.CLK
    connect Reset -> Counter.CLR_N
    connect Counter.Q[3:0] -> LED[3:0].A
}

Component should be:

- Student-friendly

- AI-friendly

- Voice-friendly

- Deterministic

- Human-readable

- Git-friendly

- Structurally mappable to visual blocks

- Exportable to SystemVerilog and Verilog

The language should borrow selected structural ideas from SystemVerilog, including:

- Modules

- Typed ports

- Packed vectors

- Named connections

- Interfaces

- Roles similar to modports

- Hierarchical instances

The initial language should avoid unnecessary procedural and verification features.

## 6. Physical Pin Identity

Every physical pin has both semantic and physical identities.

Example:

Counter.CLK
Counter.CLK@2
Counter.pin2

These references may resolve to the same physical endpoint.

The canonical model should retain:

- Instance identity

- Logical port name

- Physical package

- Physical pin number

- Direction

- Electrical behavior

- Aliases

Logical identity is primary:

Counter.CLK

Physical binding is package-specific:

DIP16 pin 2

User-facing engineering displays should normally show both:

CLK (Pin 2)

## 7. Board Model

Board is a presentation model, not a circuit model.

Example:

board CounterLearning {
    place Counter center
    stack LED[3:0] vertical right-of Counter

    bundle Counter.Q[3:0] as COUNT
    route COUNT -> LED[3:0] fanout

    show pin names
    show pin numbers
    hide power wires
}

A Component may have multiple Boards.

Examples:

learning.board
engineering.board
breadboard.board
timing.board
debug.board
presentation.board

All Boards reference the same Component design.

## 8. Headless Platform

Components Platform must function without a graphical interface.

Required access methods may include:

CLI
Local API
Remote API
MCP
Python SDK
JSON operations
Package input/output

Example CLI:

components check counter-demo.cpkg
components simulate counter-demo.cpkg
components export counter-demo.cpkg --to systemverilog
components inspect counter-demo.cpkg

A visual Board is only one client among many.

## 9. Operation Protocol

Every edit from mouse, pen, voice, text, AI, CLI, or Python must resolve into a structured operation.

Example:

{
  "domain": "component",
  "action": "connect",
  "source": "Counter.Q0",
  "target": "LED0.A"
}

Board operation:

{
  "domain": "board",
  "action": "place",
  "target": "LED0",
  "relative_to": "Counter",
  "position": "right"
}

Resource operation:

{
  "domain": "resources",
  "action": "add_dependency",
  "target": "74hc161",
  "version": "^1.0"
}

The protocol must support:

- Validation

- Transactions

- Undo

- Redo

- Replay

- Revision control

- Collaboration

- AI tool calling

- Offline operation

- Conflict detection

## 10. JSON and Python

JSON operations are the canonical command protocol.

Python is an automation and scripting client.

Python must call the same public operation service as every other client.

Allowed:

project.component.connect(
    "Counter.Q0",
    "LED0.A",
)

Not allowed:

project._graph.nodes["Counter"]["pins"][0] = ...

Python must not bypass:

- Validation

- History

- Permissions

- Revisions

- Undo and redo

- Package rules

## 11. Board Clients

A Board is a presentation and interaction client for Components.

Possible Board implementations include:

- Web Board

- Tablet Board

- Desktop Board

- VS Code Board

- Jupyter Board

- Read-only Board

- Teaching Board

- Debug Board

- Breadboard Board

- Terminal Board

- AR or VR Board

Boards may be web-based, native, embedded, or third-party.

No Board owns the package.

## 12. Tablet-First Experience

The primary visual Board should be designed tablet-first.

Desktop support should expand the workspace rather than replace the tablet interaction model.

Primary interaction methods include:

- Touch

- Pen

- Mouse

- Keyboard

- Voice

- AI assistance

Core principles:

- Content first

- Minimal chrome

- Direct manipulation

- Few permanent tools

- Searchable commands

- No ribbon interface

- No deeply nested menus

- Multiple Boards may be opened simultaneously

- Multiple windows and screens must be supported

A desktop environment may display:

Screen 1: Learning Board
Screen 2: Breadboard Board
Screen 3: Component source or simulation
Tablet: Inspector or command interface

## 13. AI Router

AI is not embedded as a mandatory dependency of Components Platform.

An AI Router is an independent client or provider layer.

It translates:

Natural language
Voice
High-level intent

into:

Components operations
Queries
Validation requests
Simulation requests
Explanations

AI providers may be:

- Local

- Cloud-based

- School-hosted

- User-selected

- Offline

- Commercial

- Open source

Components Platform must not depend on a specific AI provider.

## 14. AI Safety and Control

AI actions should support three modes:

### Inspect

Read and explain without changing the package.

### Suggest

Return a proposed transaction or patch without applying it.

### Apply

Apply an approved transaction.

AI-generated changes must be:

- Structured

- Validated

- Attributed

- Reversible

- Recorded in history

- Reviewable

AI should never directly edit internal platform files.

## 15. Team and Collaboration

Team support should be designed architecturally before implementation.

The initial implementation may remain single-user.

Every operation should still contain:

- Operation ID

- Session ID

- Author

- Agent

- Timestamp

- Base revision

A session may represent:

- A human user

- An AI agent

- A script

- A CLI process

- An auto-layout service

- A Board client

Future collaboration may add:

- Workspaces

- Roles

- Permissions

- Shared packages

- Git workflows

- Classroom workflows

- Real-time collaboration

The core platform should not require redesign when collaboration is added.

## 16. Export

Component designs should export independently of Board presentation.

Initial export targets:

- Structural SystemVerilog

- Verilog-2001

- Canonical JSON IR

- Package snapshot

Future targets may include:

- Synthesizable RTL

- Testbench generation

- Documentation

- SVG diagrams

- Netlists

- FPGA projects

- PCB-oriented formats

Board files must not be required for structural HDL export.

## 17. SystemVerilog Relationship

Component is not a replacement for SystemVerilog.

Component is a physical-component-aware, student-friendly structural language that maps cleanly to SystemVerilog.

Example mapping:

Component component      → SystemVerilog module
Component chip           → module instance
Component input/output   → module port
Component bus            → packed vector
Component connection     → named port connection or net
Component interface      → SystemVerilog interface
Component role           → modport-like structure
Package pin binding      → metadata or export annotation

Component remains simpler and more conversational than SystemVerilog.

## 18. Source of Truth

The sources of truth are:

Component
Boards
Resources
Package metadata

Derived artifacts are not authoritative.

Derived artifacts include:

- Generated Verilog

- Generated SystemVerilog

- Render cache

- Simulation cache

- Layout cache

- Dependency lock resolution

- Diagnostics indexes

Generated outputs may be deleted and recreated.

## 19. Replaceability Principle

Everything is replaceable except the Component model and Package contract.

Replaceable elements include:

- Board implementation

- Renderer

- UI framework

- Programming language

- AI provider

- Storage provider

- Simulation backend

- Export implementation

- Operating system

Stable contracts must remain independent of implementation choices.

## 20. Architectural Boundaries

### Components Platform owns

- Semantics

- Package validation

- Pin resolution

- Connectivity

- Simulation

- Export

- Revision history

- Operation application

### Board owns

- Presentation

- Interaction

- Gestures

- Local selection

- Pan and zoom

- Rendering

- Window arrangement

### AI Router owns

- Natural-language interpretation

- Voice interpretation

- Intent planning

- Tool selection

- Explanation style

### Storage Provider owns

- Persistence

- Synchronization

- Remote storage

- Git integration

- Cloud integration

No layer should silently duplicate another layer’s responsibility.

## 21. Architecture Principles

- Components is a platform, not an application.

- Components owns the truth; Boards own the experience.

- A Board is replaceable; a Package is portable.

- Every client speaks through the same operation model.

- Component describes meaning; Board describes presentation.

- Visual resources may not redefine electrical truth.

- AI is a client, not a core dependency.

- Every action must be structured, validated, attributable, and reversible.

- The platform must work offline.

- The platform must remain useful without AI.

- Tablet-first does not mean tablet-only.

- Generated artifacts are never primary sources.

- No front-end-only circuit semantics.

- Physical pin numbers and logical pin names must remain linked.

- Architecture and contracts come before implementation.